MODEL QUESTION PAPER

| Code No:CT4524 | SRGEC-R23 |
| --- | --- |
| III B.Tech I Semester Regular Examinations |  |
| COMPUTER NETWORKS (Computer Science and Engineering, Artificial Intelligence and Machine Learning & Information Technology) |  |
| Time: 3 Hours Max. Marks: 70 |  |

Note:	All questions from PART-A are to be answered in one place.

Answer all questions from PART-B. All Questions carry equal marks.

PART – A

10 × 2 = 20M

| 1. | Define LAN and WAN. |
| --- | --- |
| 2. | Mention any two characteristics of twisted-pair cable. |
| 3. | Write one difference between Go-Back-N and Selective Repeat ARQ. |
| 4. | List any two functions of error control in the data link layer. |
| 5. | Differentiate between Pure ALOHA and Slotted ALOHA. |
| 6. | Compare TDMA and CDMA in one point each. |
| 7. | What is the shortest path routing principle? |
| 8. | Name any two congestion control algorithms |
| 9. | What is the purpose of window size in TCP? |
| 10. | List any two email security mechanisms. |

PART – B

5 × 10 = 50M

Unit - I

|  |  | BL |
| --- | --- | --- |
| 2. | a) Compare the OSI and TCP/IP reference models in detail. (5M) b) Discuss the features, advantages, and limitations of fiber optic communication. (5M) | L4 |
|  | (OR) |  |
| 3. | a) Explain the structure and functions of each layer in the OSI model. (5M) b) Analyze the role of physical media in data transmission. (5M) | L2 |
| Unit - II |  |  |
| 4. | a) Explain the operation of the bit-oriented protocol HDLC with the required frames. (5M) b) Data word = 10011010, Generator polynomial = x³ + x² + 1 (binary 1101). Compute the CRC codeword. (5M) | L3 |
|  | (OR) |  |
| 5. | a) Encode the 4-bit data 1011 using (7, 4) Hamming Code with even parity. (5M) b) Discuss the elementary data link protocols with diagrams (Simplex, Stop-and-Wait, Noisy Channel). (5M) | L3 |
| Unit - III |  |  |
| 6. | a) Discuss the working of CSMA with Collision Detection and Collision Avoidance.(5M) b) Compare FDMA, TDMA, and CDMA in terms of bandwidth utilization, complexity, and applications. (Analyzing) (5M) | L2 |
|  | (OR) |  |
| 7. | a) Analyze the efficiency of ALOHA protocols in network communication. (5M) b) Describe the evolution of Ethernet from Standard to 10 Gigabit Ethernet. (5M) | L4 |
| Unit - IV |  |  |
| 8. | a) Explain the Distance vector routing algorithm with an numerical example. (5M) b) Discuss about concept of leaky bucket algorithm with a neat diagram. (5M) | L3 |
|  | (OR) |  |
| 9. | a) Compare IPv4 and IPv6 in terms of addressing, header size, and features. (5M) b) Describe Dijkstra shortest path algorithm. Also show working of Dijkstra algorithm with the help of an example. (5M) | L4 |
| Unit - V |  |  |
| 10. | a) Explain the working of TCP with reference to segments, windows, and flow control. (5M) b) Analyze the role of DNS in internet communication. (5M) | L4 |
|  | (OR) |  |
| 11. | a) Describe the architecture and working of HTTP and email systems. (5M) b) Describe congestion control in TCP using Slow Start and Congestion Avoidance. (5M) | L2 |